Confined Space: Forced Air Ventilation Operation Quiz

Student:  _____________________________________________		Date: ______________

Minimum passing score is 80%.

- What is the safety industry’s recommended minimum number of air changes per hour during a confined space entry?

  - 2					c. 10

  - 5					d. 20

- What is mechanically pushing air that is potentially contaminated from a confined space called?

  - Forced air ventilation		c. Exhausted air ventilation

  - Blower air ventilation		d. Negative air ventilation

- How does attaching ducts or elbows to the blower affect the volume of air flow from the blower?

  - Reduces air flow			c. Has no impact on air flow

  - Increases air flow			d. Can increase or decrease, depends on the duct or elbow

- Continuous forced air ventilation is required for permit-required confined space entries for what conditions?

  - Actual hazardous atmospheres		c. Immediately dangerous to life or health (IDLH) atmospheres

  - Potential hazardous atmospheres		d. All of the above

- When can forced air ventilation operations be stopped during a permit-required confined space entry?

  - The last entrant has left the space	c. Oxygen levels are between 19.5 & 23.5

  - Lower Explosive Limit is 0.0		d. When the last Lock Out Tag Out device is removed

- The forced air blower duct in a vertical confined space, such as a below-grade vault, should extend to what level in the space?

  - Floor-level				c. Workers’ breathing zone (nose) level

  - Mid-height level			d. Within 1 foot of ceiling

- What is the volume of a confined space vault that measures 30 feet long, 20 feet wide, & 8 feet high?

  - 58 cu. ft.				c. 4,800 cu. ft.

  - 408 cu. ft. 			d. 50, 800 cu. ft.

- What is the total cubic feet per hour (cfh) delivery rate of air needed to achieve 20 air changes per hour (ACH) for a confined space that measures 20 ft. x 10 ft. x 10 ft.?

  - 2,000 cfh				c. 10,000 cfh

  - 8,000 cfh				d. 40,000 cfh

- For the above confined space operation, what would be the minimum cubic feet per minute (cfm) needed at the end of the forced air blower system?

  - 450 cfm				c. 670 cfm

  - 540 cfm				d. all meet the minimum requirement

- If the blower stops working during a confined space entry, what must be the response?

  - Immediately evacuate entrants	c. Verify air quality is still safe.  Evacuate entrants if unsafe.

  - Call the Entry Supervisor		d. Put the confined space rescue team on stand-by

Confined Space: Forced Air Ventilation Equipment Skill Sheet

Student:  _____________________________________________		Date: ______________

Evaluator: ____________________________________________		 Pass      Fail

| Equipment Used for Observation: | Make | Model |
| --- | --- | --- |
| Ventilation Blower & Duct |  |  |

______ (initial) I have reviewed the Owners/Operator’s manual for the above ventilation equipment and been given an opportunity to ask questions.

| Required Knowledge: |  |
| --- | --- |
| The student shall demonstrate they have the necessary information to safely operate the above blower, duct and accessories to the satisfaction of the evaluator. |  |
| Describes the proper clean air intake considerations for placement of blower | Pass Fail |
| Can give the rated flow rate of the blower and effective flow rate with duct(s), elbows, and other accessories connected | Pass Fail |
| Can describe the advantages and methods to achieve effective air movement in and through a confined space. | Pass Fail |
| Given the dimensions of a confined space, the student can give the time needed to achieve the needed number of air changes using the blower and attachments. | Pass Fail |
| Can describe the benefits of positive ventilation and negative ventilation and the best use of each. | Pass Fail |
| Can explain how vapor density of gases can affect the efficacy of removing gases from a confined space. | Pass Fail |

| Demonstrated Skill: Monitoring the atmosphere in a confined space |  |
| --- | --- |
| The student shall ventilate a confined space with 100% accuracy using the air blower and accessories. |  |
| Student inspects the blower and accessories for damage before use. | Pass Fail |
| Student places blower on a stable surface and upwind from the work location, with a clean source of air. | Pass Fail |
| Student sets up for a positive pressure (forced air) ventilation operation - Connects the duct and accessories to the blower’s outlet - Connects duct and elbows, as necessary, to position cone of air flow that completely covers the entrance opening - Establishes an exhaust opening | Pass Fail |
| Student sets up for a negative exhaust ventilation operation - Connects the duct and accessories to the blower’s inlet - Connects duct and elbows, as necessary, to effectively extract hazardous atmospheric contaminants from a simulated welding operation. | Pass Fail |
| Student connects the blower to a proper electrical source with a GFCI. | Pass Fail |
| Student checks for proper operation of blower and sufficient air movement | Pass Fail |
| Student completes air ventilation portion of Confined Space Permit. | Pass Fail |
| Student breaks down the blower and accessories, and returns unit to storage. | Pass Fail |

Comments:

Confined Space: Forced Air Ventilation Operation Quiz – Answer Key

Student:  _____________________________________________		Date: ______________

Minimum passing score is 80%.

- What is the safety industry’s recommended minimum number of air changes per hour during a confined space entry?

  - 2					c. 10

  - 5					d. 20

- What is mechanically pushing air that is potentially contaminated from a confined space called?

  - Forced air ventilation		c. Exhausted air ventilation

  - Blower air ventilation		d. Negative air ventilation

- How does attaching ducts or elbows to the blower affect the volume of air flow from the blower?

  - Reduces air flow			c. Has no impact on air flow

  - Increases air flow			d. Can increase or decrease, depends on the duct or elbow

- Continuous forced air ventilation is required for permit-required confined space entries for what conditions?

  - Actual hazardous atmospheres		c. Immediately dangerous to life or health (IDLH) atmospheres

  - Potential hazardous atmospheres		d. All of the above

- When can forced air ventilation operations be stopped during a permit-required confined space entry?

  - The last entrant has left the space	c. Oxygen levels are between 19.5 & 23.5

  - Lower Explosive Limit is 0.0		d. When the last Lock Out Tag Out device is removed

- The forced air blower duct in a vertical confined space, such as a below-grade vault, should extend to what level in the space?

  - Floor-level				c. Workers’ breathing zone (nose) level

  - Mid-height level			d. Within 1 foot of ceiling

- What is the volume of a confined space vault that measures 30 feet long, 20 feet wide, & 8 feet high?

  - 58 cu. ft.				c. 4,800 cu. ft.

  - 408 cu. ft. 			d. 50, 800 cu. ft.

- What is the total cubic feet per hour (cfh) delivery rate of air needed to achieve 20 air changes per hour (ACH) for a confined space that measures 20 ft. x 10 ft. x 10 ft.?

  - 2,000 cfh				c. 10,000 cfh

  - 8,000 cfh				d. 40,000 cfh

- For the above confined space operation, what would be the minimum cubic feet per minute (cfm) needed at the end of the forced air blower system?

  - 450 cfm				c. 670 cfm

  - 540 cfm				d. all meet the minimum requirement

- If the blower stops working during a confined space entry, what must be the response?

  - Immediately evacuate entrants	c. Verify air quality is still safe.  Evacuate entrants if unsafe.

  - Call the Entry Supervisor		d. Put the confined space rescue team on stand-by