| Shooting Range Facilities Hazard Identification Inspection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Agency: |  |  |  |  | Inspection Date: |
| Name of Inspector: |  |  |  |  | Title: |
| Location Inspected: |  |  |  |  |  |
| # | Needs Work | N/A | OK | Condition Surveyed |  |
|  | Physical Facilities |  |  |  |  |
| 1 |  |  |  | Each firing lane is approximately 4 feet wide. |  |
| 2 |  |  |  | Pipes, conduits, and other projecting surfaces are baffled or covered by a material that will protect these items and prevent ricochets. |  |
| 3 |  |  |  | No windows or doors are located in front of the firing line. (Except access door to the back of the bullet trap). |  |
| 4 |  |  |  | There are no open floor drains in the range. |  |
| 5 |  |  |  | Pipes, conduits and walls are sealed to prevent leakage of lead dust from the range into other areas |  |
| 6 |  |  |  | There is no carpet, drapes or other fiber-like material in the range. |  |
| 7 |  |  |  | The interior surfaces or the range floor, walls and ceiling are smooth with no protruding edges or devices |  |
| 8 |  |  |  | The roof provides ballistic security. |  |
| 9 |  |  |  | The walls provide ballistic security. |  |
| 10 |  |  |  | Interior mortar joints are flush with the interior surface. |  |
| 11 |  |  |  | The plenum wall is adequately supported and thick enough to avoid flexing. |  |
| 12 |  |  |  | The entrance door to the range is weather-stripped. |  |
|  | Lighting |  |  |  |  |
| 13 |  |  |  | Lighting is uniform, non-glaring and does not cause shadows. |  |
| 14 |  |  |  | Illumination is at least 75-foot candles on the targets and 30-foot candles in all other areas. |  |
| 15 |  |  |  | All lighting is protected by baffles and placed so that the firer has an unobstructed view downrange. |  |
| 16 |  |  |  | Downrange lighting begins approximately 18 feet from the firing line and ends approximately 8 feet from the target line. |  |
| 17 |  |  |  | Emergency lights are provided behind the firing line and are in working condition. |  |
| 18 |  |  |  | Exit lights are provided and working as required. |  |
| 19 |  |  |  | Lighting of at least 30-foot candles is provided behind the bullet trap for maintenance. |  |
| 20 |  |  |  | No known electrical hazard exists in the range. |  |
|  | Bullet Traps |  |  |  |  |
| 21 |  |  |  | A bullet trap is permanently installed in the range. |  |
| 22 |  |  |  | The bullet trap is installed per manufacturer instructions. |  |
| 23 |  |  |  | Bullet traps are of a commercial design. |  |
| 24 |  |  |  | The thickness of plate/depth of rubber type bullet trap must be adequate to attenuate the maximum caliber of ammunition authorized to be fired on the range. |  |
| 25 |  |  |  | Plate/rubber type backstops extend to a point directly below the leading edge of the sloped plate |  |
| 26 |  |  |  | Forward edges in a louver or venetian blind type bullet trap are maintained in a knife edge condition to prevent ricochets |  |
| 27 |  |  |  | Steel bullet traps are not bowed, punctured or severely pitted. |  |
|  | Targets |  |  |  |  |
|  |  |  |  | A target retrieval system is operable at all lanes. (Any one firing lane without a retrieval system will not be used for firing.) |  |
| 24 |  |  |  | The target retrieval system is constructed in such a manner as to minimize flat surfaces exposed to the firing line. |  |
| 25 |  |  |  | Only authorized paper targets are used in the range. |  |
|  | Maintenance |  |  |  |  |
| 26 |  |  |  | Dry sweeping does not occur in the range. |  |
| 27 |  |  |  | No brooms are located in the range. |  |
| 28 |  |  |  | A range custodian or commercial contractor is appointed for the range that is fully trained and aware of his/her responsibilities. |  |
|  | Personal Protective Equipment |  |  |  |  |
| 29 |  |  |  | All personnel in the range during firing wear ANSI approved eye protection. |  |
| 30 |  |  |  | All personnel in the range during firing wear ANSI approved hearing protection. |  |