BLEACHER & GRANDSTAND INSPECTION CHECKLIST

| Facility Name: | ____________________________ | Location / Address: | ____________________________ |
| --- | --- | --- | --- |
| Bleacher ID / Unit #: | ____________________________ | Bleacher Type: | ____________________________ |
| Number of Rows: | ____________________________ | Inspection Date: | ____________________________ |
| Inspector Name: | ____________________________ | Inspection Type: | ____________________________ |
| Weather Conditions: | ____________________________ | Last Inspection Date: | ____________________________ |

Bleacher Type: Permanent / Portable / Telescopic / Temporary     |     Inspection Type: Pre-Season / Mid-Season / Post-Season / Post-Storm / Routine

SECTION 1: STRUCTURAL INTEGRITY

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| No loose or missing hardware on cross-braces and frame connections | ☐ | ☐ | ☐ |  |
| No cracked or broken welds | ☐ | ☐ | ☐ |  |
| Bleacher is solidly anchored or attached to ground surface | ☐ | ☐ | ☐ |  |
| No bent or warped benches or components | ☐ | ☐ | ☐ |  |
| No sharp edges from damage | ☐ | ☐ | ☐ |  |
| No missing or damaged safety end caps | ☐ | ☐ | ☐ |  |
| No rust or signs of significant corrosion | ☐ | ☐ | ☐ |  |
| No signs of environmental stress damage (rot, UV degradation, etc.) | ☐ | ☐ | ☐ |  |
| Identification tag present with installation and inspection information | ☐ | ☐ | ☐ |  |
| Structural condition acceptable (no signs of imminent failure or collapse risk) | ☐ | ☐ | ☐ |  |

SECTION 2: GUARDRAILS & FALL PROTECTION

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Guardrails present on backs and open ends where footboard/seatboard/aisle is 30 inches or more above ground | ☐ | ☐ | ☐ |  |
| Guardrail top surface is at least 42 inches above leading edge of footboard, seatboard, or aisle | ☐ | ☐ | ☐ |  |
| All guardrail openings prevent passage of a 4-inch diameter sphere (up to 34 inches height) | ☐ | ☐ | ☐ |  |
| Openings between 34 and 42 inches height prevent passage of an 8-inch diameter sphere | ☐ | ☐ | ☐ |  |
| Guardrail infill uses vertical members only; no ladder-like patterns | ☐ | ☐ | ☐ |  |
| Any foothold openings in guardrail infill are no wider than 1.75 inches | ☐ | ☐ | ☐ |  |
| If chain-link fencing used: mesh size is 1.25 inches square or smaller | ☐ | ☐ | ☐ |  |
| Guardrails are free of damage, bending, or looseness | ☐ | ☐ | ☐ |  |
| Where bleachers are adjacent to a wall: 4-inch sphere cannot pass between bleachers and wall | ☐ | ☐ | ☐ |  |

SECTION 3: SEATING COMPONENT OPENINGS

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Openings between footboard and seatboard prevent passage of a 4-inch diameter sphere (where 30+ inches above grade) | ☐ | ☐ | ☐ |  |
| Openings between seatboard and riser prevent passage of a 4-inch diameter sphere (where 30+ inches above grade) | ☐ | ☐ | ☐ |  |
| No openings that would permit a fall of 30 inches or more | ☐ | ☐ | ☐ |  |
| Aisle accessways more than 30 inches above grade: openings prevent passage of a 4-inch diameter sphere | ☐ | ☐ | ☐ |  |
| Horizontal gaps between footboards and seatboards do not exceed 0.25 inch | ☐ | ☐ | ☐ |  |

SECTION 4: EGRESS & ACCESSIBILITY

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Aisles are clear of obstructions | ☐ | ☐ | ☐ |  |
| Minimum aisle width is maintained per applicable standards | ☐ | ☐ | ☐ |  |
| Stepped aisle tread depth is at least 11 inches | ☐ | ☐ | ☐ |  |
| Stepped aisle riser height is between 4 and 8 inches | ☐ | ☐ | ☐ |  |
| Tread and riser dimensions are uniform throughout (tolerance ≤ 0.375 inch) | ☐ | ☐ | ☐ |  |
| Row-to-row spacing is at least 22 inches | ☐ | ☐ | ☐ |  |
| Minimum seat depth is 9 inches | ☐ | ☐ | ☐ |  |
| For exterior installations: travel distance from any seat to perimeter does not exceed 400 feet | ☐ | ☐ | ☐ |  |
| Exit signage is visible and in good condition (where applicable) | ☐ | ☐ | ☐ |  |
| Accessible routes and seating areas are unobstructed | ☐ | ☐ | ☐ |  |

SECTION 5: HANDRAILS

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Handrails present on all required stepped aisles | ☐ | ☐ | ☐ |  |
| Handrail height is 34 to 38 inches above tread nosings | ☐ | ☐ | ☐ |  |
| Mid-aisle handrails are discontinuous at intervals not exceeding 5 rows | ☐ | ☐ | ☐ |  |
| Gaps in mid-aisle handrails are between 22 and 36 inches wide | ☐ | ☐ | ☐ |  |
| Handrail gripping surfaces are continuous and free of obstructions | ☐ | ☐ | ☐ |  |
| Handrails are free of sharp or abrasive elements | ☐ | ☐ | ☐ |  |
| Clear space between handrail and wall/surface is at least 1.5 inches | ☐ | ☐ | ☐ |  |
| Handrails are structurally sound and firmly attached | ☐ | ☐ | ☐ |  |

SECTION 6: CAPACITY & SIGNAGE

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Maximum occupancy/load capacity is posted on the bleacher unit | ☐ | ☐ | ☐ |  |
| Identification tag is present and legible (includes installation and inspection info) | ☐ | ☐ | ☐ |  |
| Any required warning notices are posted (for units not yet certified as compliant) | ☐ | ☐ | ☐ |  |
| No visible overcrowding or overloading conditions | ☐ | ☐ | ☐ |  |

SECTION 7: PORTABLE / TEMPORARY BLEACHERS  (Complete if applicable)

| Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- |
| Bleacher is set up on stable, level ground | ☐ | ☐ | ☐ |  |
| Anchoring and stabilization confirmed per manufacturer specifications | ☐ | ☐ | ☐ |  |
| No damage from transport or previous setup/breakdown cycles | ☐ | ☐ | ☐ |  |
| Manufacturer's rated capacity is not exceeded | ☐ | ☐ | ☐ |  |
| Setup verified by trained personnel prior to use | ☐ | ☐ | ☐ |  |

SECTION 8: OVERALL ASSESSMENT

| Overall Condition Rating: | ☐ Satisfactory ☐ Requires Monitoring ☐ Requires Repair Before Next Use ☐ Remove from Service Immediately |
| --- | --- |
| Bleacher Approved for Use: | ☐ Yes ☐ No |
| If No — Date Closed/Tagged Out: | ___________________________ |

DEFICIENCIES NOTED

Complete for each deficiency identified. Attach additional sheets if needed.

| Item # | Description of Deficiency | Location on Structure | Severity (L/M/H) | Corrective Action Required | Assigned To | Target Completion | Date Resolved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |

INSPECTOR CERTIFICATION

"I certify that this inspection was conducted in accordance with applicable standards and that the findings recorded above are accurate to the best of my knowledge."

| Inspector Signature: | ______________________________ | Date: | ______________________________ |
| --- | --- | --- | --- |
| Print Name: | ______________________________ | Title: | ______________________________ |
| Supervisor Review (if required): | ______________________________ | Date: | ______________________________ |

NOTES / ADDITIONAL OBSERVATIONS

______________________________________________________________________________________________________________

______________________________________________________________________________________________________________

______________________________________________________________________________________________________________

______________________________________________________________________________________________________________

______________________________________________________________________________________________________________

______________________________________________________________________________________________________________

REFERENCES

New Jersey Standard on Bleachers, Folding and Telescopic Seating, and Grandstands 2017 (N.J.A.C. 5:23), based on ICC 300-2017

CPSC Guidelines for Retrofitting Bleachers

ICC 300-2017: Standard for Bleachers, Folding and Telescopic Seating, and Grandstands