Cascade System SCBA Multi-Cylinder Refill Skill Demonstration

Candidate: ______________________________	Evaluator: ______________________________ 		Date: _____________

_________  (Candidate initial) I have been given the opportunity to review the cascade system’s Owner’s Manual.

| Skill | S: Satisfactory / U: Unsatisfactory | Comments |
| --- | --- | --- |
| Candidate conducts pre-use inspection of cascade system in accordance with manufacturer’s instructions. |  |  |
| Inspects each cylinder for physical damage (dents, gouges, corrosion, heat damage) and is secured from tipping. |  |  |
| Checks the hydrostatic test date on each cylinder. |  |  |
| Confirms relief valves are present and unobstructed |  |  |
| Inspects gauges, manifold and fill hoses for damage, cracks, or wear. |  |  |
| Checks all O-rings and sealing surfaces. Replaces damaged O-rings before use |  |  |
| Candidate inspects and prepares the SCBA cylinder for refilling. |  |  |
| Inspects SCBA cylinder to be filled for damage before connecting. |  |  |
| Checks SCBA cylinder pressure and last hydrostatic test date. Does not refill cylinders beyond hydrostatic test interval. |  |  |
| Inspects fill hose connection and O-ring before attaching |  |  |
| Selects correct fill hose/adapter for SCBA cylinder connection |  |  |
| Candidate demonstrates the proper cascade system process for refilling SCBA cylinders. |  |  |
| Connects cascade fill hose to SCBA cylinder — hand-tight, then snug. (no over-torque) |  |  |
| Opens first cylinder in cascade system valve SLOWLY and in a controlled manner. Start with the lowest-pressure storage cylinder (or bank) first. Then progress sequentially to higher-pressure cylinders to finish or top off the bottle. |  |  |
| Monitors pressure gauge during fill — does not leave fill unattended. Stops fill when pressures equalize or SCBA cylinder reaches rated pressure — does not overfill. |  |  |
| Closes cascade cylinder valve completely before switching cylinders. |  |  |
| Bleeds pressure from fill hose before disconnecting (uses bleed valve or controlled release). |  |  |
| Switches to next highest pressure cascade cylinder and repeats process. |  |  |
| Continues cascade sequence until SCBA cylinder is filled to rated pressure. |  |  |
| Closes all cascade cylinder valves after filling is complete. |  |  |
| Disconnects fill hose from SCBA cylinder safely. |  |  |
| Verifies SCBA cylinder pressure on SCBA gauge after fill. |  |  |
| Candidate completes post-fill documentation and prepares cascade system and area for next use. |  |  |
| Documents fill in the cascade system log. (date, time, cylinders used, pressures) |  |  |
| Stows fill hose properly — no kinks, coiled and secured. |  |  |
| Identifies any cascade cylinders that need recharging (below minimum pressure). |  |  |
| Tags out any cylinder that is damaged, out of hydrostatic test date, or below minimum pressure. |  |  |
| Candidate demonstrates or explains their response to cascade system malfunctions or emergency situations |  |  |
| Correctly identifies and responds to a leaking connection. (isolates, does not attempt to tighten under pressure) |  |  |
| Correctly identifies a damaged cylinder and removes it from service. (tags out, does not use) |  |  |
| Explains what to do if relief valve activates. |  |  |
| Knows who to notify in case of system malfunction or emergency. |  |  |

Evaluator’s signature: __________________________________________________________________________	Date: __________________